Three-Channel Trickle-Irrigation-Controller

1 Features	1

2 Getting started	1

2.1 Menu 1 = Pump 1 Starting Time and Running Time	1

2.2 Menu 2 = Pump 2 Starting Time and Running Time	2

2.3 Menu 3 = Pump 3 Starting Time and Running Time	2

2.4 Menu 4 = Set-value for the Moisture-Sensors	2

2.5 Menu 7 = Input of Time	2

2.6 Menu 8 = Battery-voltage and output-voltage	2

2.7 Menu 9 = Status of output and manual mode	2

2.8 If you want to have some NiCd or NimH Accumulator or a battery to be reloaded	2

2.9 If you need voltage for a Walkman and / or Radio and / or CD-Player	2

2.10 More automatic Functions and messages	3

2.11 Problems:	3

2.12 Hints	3

3 Appendix	3

3.1 Where to put the devices?	3

3.2 Connection of the cables	3

3.3 Update of the software	5

3.4 For professionals: Supervising of the values with the PC	5

# Features

- three independent voltage-sources, one with 8V, one switchable and one adjustable for 	small-appliances like CD-Player, Walkman

- Micro-Controller controlled Quick-charger for NiCd. - and NiMH-Akkumulator    with 	Discharge-function

# Getting started

First you have to get the cables connected according the Appendix.

After switching on at the left switch, you see in the display the “Ground-Menu”: The actual-value of the three humid-sensors are shown in this menu; If you press “#” in this menu, then you can select between the display of the time or the battery-voltage;

From the Ground-Menu (the menu on the display after being switch on) you can select different Submenus by pressing the key 1 to 9.

## Menu 1 = Pump 1 Starting Time and Running Time

If you select Nr. 1 from the Ground-Menu, then you can input the Start-Time for Pump 1 (or device 1). First you give the hour (0-23) and then press “#” that you can input the minute you want to have the pump been startet. With the next “#” you can say how long you want to have the pump running. If you enter “0” then the pump will never been startet; If you give 255 the pump will allways be on.

If you press “#” again you can realize a kind of triggle irrigation. Triggle irrigation is a very efficient way of irrigation, you need much less water. But the valves for to realize this is not very cheap. With this you can realize a simple “triggle-irrigation” by switching the pump on and off in short Intervalls;

If you input 1 for example for “all..”; That means every minute the pump will run as long as you wrote in (on...); But this is in second now!

One Example: If you want to have the pump all ten minutes and for 30 Seconds on: You input 30 (Second) at “on” and 10 (Minutes) at “all”;

If you input 102 for “all.. “ that means every second hour the pump is on for the time you wrote in “on...”; But this in minutes now!; One Example: every third hour for 10 minute: Input of 10 for “on” and 103 (3.day) for “all”

If you input 202 for “all.. “ that means every second day the pump is on for the time you wrote in “on...”; But this in minutes now!

But the pump is not running if the humidy is too high (see Sub-Menu 4)! If you don’t use this input “100” for the Setpoint of Humid. But if there is no sensor used it automatically will be detected!

Return from the    sub-menu by pressing    “*”;

## Menu 2 = Pump 2 Starting Time and Running Time

If you select Nr. 2 from the Ground-Menu, then you can input the Start-Time for Pump 2; see Pump 1 how to input the data.

## Menu 3 = Pump 3 Starting Time and Running Time

If you select Nr. 3 from the Ground-Menu, then you can input the Start-Time for Pump 3; see Pump 1 how to input the data.

## Menu 4 = Set-value for the Moisture-Sensors

If you select Nr. 4 from the Ground-Menu, then you can input the set-values for Moisture for the three pumps; The value you enter here surpresses the start of the irrigation, if the humidy is too high. That means if you input for example 60% and the acual value is 70% then there will be no irrigation until it falls unter 60%.

If you press “#” you can input the moisture for Pump 2 and 3; Return from the    sub-menu by pressing    “*”;

## Menu 7 = Input of Time

If you select Nr. 7 from the Ground-Menu, then you can Input the actual time. First give the hour (0 to 23), then push “#” and when the flashing “?” is at the next figure give the minutes. Finish with “#”; After the voltage has been completely off, you have to Input the time again. Return from the    sub-menu by pressing    “*”;

## Menu 8 = Battery-voltage and output-voltage

If you select Nr. 8 from the Ground-Menu, then you can see the Battery-voltage and output-voltage. The output-voltage is the voltage 2 and it is adjusted at the backside of the device. The output of this voltage is at the terminal 1 and 2 at the backside. Return from the    sub-menu by pressing    “*”;

## Menu 9 = Status of output and manual mode

If you select Nr. 9 from the Ground-Menu, then you see which output is on (1) and which is off (0);

The output mean:

1= (P1) Pump 1;

2= (P2) Pump 2;

3= (P3) Pump 3;

4= (P4) Pump 4;    this pump cannot run independent but is automatic startet when pump 3 is startet

5= (Bo) Battery O.K.; (output voltage, NiCd loading active)

6= (Bf) Battery full, loading finished (no more loading from Solar/Transformer)

If you press 1 till 6 then you manually can switch on and off the output and the manual mode is active. That means that no more output is switched on automatically

## If you want to have some NiCd or NimH Accumulator or a battery to be reloaded

Easily put the Accumulator in the according fold (look for + and Minus!, the spring is always Minus!), put the switch at the top-Side of the device to “NiCd” and then they will be Quick-charged automatic (in 2 or 3 hours). Loading is finished when the green light is off;

For NiCd-Accumulator it is better to press the “Refresh-Button” after putting them to    the fold. Then the yellow lamp will be on for some time and they will be discharged before being charged. This gives more Capacity to the NiCd-cells. Use this only for NiCd-Cells.

You can charge normal batteries as well if they are not completely discharged.    This may be possible up to ten times; In this case turn the switch at the top-side of the device to “battery” and then put the batteries to the folder.

## If you need voltage for a Walkman and / or Radio and / or CD-Player

Select the voltage you need for the device on the switch on the right side;

You can select between following voltages:    1= 3V; 2= 4.5V; 3= 6V; 4= 8V; 5= 9V; 6= 11Volt

Walkman use 3V, CD-Player mostly 8V; For the radio etc. you have to look;

Switch to the right voltage before you connect the device at the terminal 3 (“+”) and 4 (Minus) at the backside of the device. There will be a cable available for doing this (white has to go to +!) //;

If you need 8V you can directly use the terminal 5 (“+”) and 6 (Minus) at the backside of the device.

If you need another voltage in between you can also use the adjustable voltage. Adjust it with the switch on the backside (described “Voltage 2”), you see the value in Sub-Menu 4; Then connect the device at terminal 1 (“+”) and 2 (Minus) at the backside.

All the voltage-outputs are independent and temperature and short-cut protected.

## More automatic Functions and messages

- if the battery will be empty soon, and no transformer is connected there will be a message. After some more time the output-voltage will be switched off and the loading of the NiCd-cells will stop. The pumping etc. is switched off, only the computer continues running, because he needs very little energy. After reloading the battery the devices are switched on again.

- if the battery is full, the loading from solar and the transformer will be finished and only a trickle-charging continues.

- if there is no more water in the container, there will be a alarm. Press “0” for to acknowledge this;

## Problems:

1. The controller is on, but there is no pump or motor on

- the output is set in Sub-Menu 9?

- the fuse on the right side is O.K.?

## Hints

- You can use this functions not only for a irrigation-system. You can switch all connected devices on and off at a time-programm;

- If you don’t use the humid-sensors, set the value to 100%; Then there is no effect from them.

-You can connect 12V and 220V Pumps, with the max. of 6A.

# Appendix

## Where to put the devices?

-The control-unit with the LCD-Display should not be put into the sun for a longer time, because this can destroy the Display. It could be in the kitchen or where the water is required. The cable and tube-bundle has to be put from the controller to the Solar-Cooker.

## Connection of the cables

There are three terminal-blocks at the device; 12 Terminals on the backside and 14 / 12 Terminals on the front-side;    Please be careful not to put two cables together because this will be a short-cut and you have to replace the fuse!

- Connection of the Transformer: The Transformer must be connected to the socket in the front side of the device.

- Connection of a lead-battery: For the pump uses normally more current than the Transformer can deliver, a battery has to be connected at terminal 11 ("-") and 12 ("+") at the lower terminal block at the front-side.            It could be for example a old lead-battery of a car or    a special solar-battery or also NiCd Batteries from a Hand-Lamp. The battery is loaded from the transformer and as well from the Solar-Module.    Overloading and unterloading of the battery is supervised. When the Battery is empty the Output-Voltages (the switchable and adjustable) and the NiCd-loading is switched off.

- Connection of Pump 1: Connect it at terminal 1 (Minus) and 2 (“+”) at the upper terminal block; if you have a 12V Pump there must be a bridge between 3 and 4;

If you have a 220V Pump there must be the 220V input-cable being connected at terminal 1 and 3 (upper terminal but from down);

- Connection of Pump 2: Connect it at terminal 6 (Minus) and 7 (“+”) at the upper terminal block; if you have a 12V Pump there must be a bridge between 4 and 5;

If you have a 220V Pump there must be a bridge between terminal 1 and 6 (and the Minus-cable in Terminal 6 must be removed!) and a bridge between terminal 3 and 5 (upper terminal but from down);

- Connection of Pump 3: Connect it at terminal 9 (Minus) and 8 (“+”) at the upper terminal block; if you have a 12V Pump there must be a bridge between 4 and 5;

If you have a 220V Pump there must be a bridge between terminal 6 and 9 (upper terminal but from down);

- Connection of Pump 4: Connect it at terminal 10 (Minus) and 11 (“+”) at the upper terminal block; if you have a 12V Pump there must be a bridge between 11 and 12; if you have a 220V Pump there must be a bridge between terminal 5 and 11 and no between 4 and 5 (upper terminal but from down);

- Connection of the sensors: the first Humid-sensor at terminal 7 and 8, the second Humid-sensor at terminal 7 and 9. the third Humid-sensor at terminal 10 and 11at the backside terminal-block.

- A Sensor for a addtional empty-signal can be connected at Terminal 10 and 12; But it should have at least 1 KOhm //; Anyway there is a automatic-detection of the empty-level and a alarm-Signal integrated

| Terminal-connections for Irrigation-System with 24V Pumps |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| front-side down |  |  | front-side up |  | backside |  |
| 1 | 12V+ |  | 1 | Minus | 1 | U_adjustable + |
| 2 | 12V+ |  | 2 | Pump 1 + | 2 | Minus |
| 3 | Solarmodule + |  | 3 | Input Voltage P1 | 3 | U_switchable.+ |
| 4 | Output Solar Relais |  | 4 | 12V+ | 4 | Minus |
| 5 | Solar switched |  | 5 | Input P2+3 | 5 | 8V + |
| 6 | Solarmodule - |  | * 6 | Minus | 6 | Minus |
| 7 | 12V switched |  | 7 | Pump 2 + | 7 | 5V + |
| 8 | Relais 1 Out |  | 8 | Pump 3 + | 8 | Sensor Humid 1 |
| 9 | Relais 1 In |  | * 9 | Minus | 9 | Sensor Humid 2 |
| 10 | 12V+ |  | 10 | Pump 4 + | 10 | 5V + |
| 11 | Batt Minus |  | 11 | Input Pump 4 | 11 | Humid 3 |
| 12 | Battery + |  | 12 | 12V+ | 12 | Empty |
| *There must be a Minus-Cable connected if you haven't removed it because of 220V Pumps |  |  |  |  |  |  |
| Terminal-connections for Irrigation-System with 220V Pumps |  |  |  |  |  |  |
| front-side down |  |  | front-side up |  | backside |  |
| 1 | 12V+ |  | 220 V in * 1 | Pump 1 a | 1 | U_adjustable + |
| 2 | 12V+ |  | 2 | Pump 1 b | 2 | Minus |
| 3 | Solarmodule + |  | 220 V in * 3 | Input Voltage P1 | 3 | U_switchable.+ |
| 4 | Output Solar Relais |  | 4 | 12V+ | 4 | Minus |
| 5 | Solar switched |  | * 5 | Input P2+3 | 5 | 8V + |
| 6 | Solarmodule - |  | * 6 | Pump 2 a | 6 | Minus |
| 7 | 12V switched |  | 7 | Pump 2 b | 7 | 5V + |
| 8 | Relais 1 Out |  | 8 | Pump 3 b | 8 | Sensor Temp_Past |
| 9 | Relais 1 In |  | * 9 | Pump 3 a / 4a | 9 | Sensor Temp_Cook |
| 10 | 12V+ |  | 10 | Pump 4 b | 10 | 5V + |
| 11 | Batt Minus |  | ** 11 | Input Pump 4 | 11 | Sensor 3 |
| 12 | Battery + |  | 12 | 12V+ | 12 | Sensor 4 |
| *There must be a bridge between terminal 1 and 6, 3 and 5 and 6 and 9; |  |  |  |  |  |  |
| and no other cable should go to this terminal exept the 220V bridge |  |  |  |  |  |  |
| ** if you use also Pump 4 then the bridge betwen 11 and 12 must be removed, and there must be a bridge between 5 and 11 |  |  |  |  |  |  |

optional:

- The Solarmodul or modules have to be connected at terminal 3 ("+") and terminal 6 ("-") on the lower terminal block at the front side.    This is optional. It is also running without Solarcell;

- Connection of small devices like Walkman etc.; This can be connected at the switchable voltage at terminal 3 (“+”) and 4 (Minus) at the backside of the device. But switch to the right voltage before connecting it!; If 8V is required it also can be connected to 5    ("+") and 6 (Minus); Or to terminal 1 (“+”) and 2 (Minus) if any other adjustable voltage is required (Corresponds to voltage 2). All voltages work independent and have max. 2A (8V only 1A);

## Update of the software

In order to reload software to the Solar-Cooker-Controller, use the cable you get and plug the side marked ”not at the PC...” in the device. The other side plug in a free input of your PC. If the according plug is occupied, remove for a short time the plug (may be the mouse). Then switch the Solar-Controller off and on. The Display will be blank after.

Now you can load different Software:

irrigate is a program for a irrigation-control

temptime is a programm for to have a Temperature-Time Programm; You can use it for to make beer in the Solar-Cooker for example

solar3 is the original version of the solarcooker-Software

solar4, solar5 etc.    is a update you get from me or from the Web-Page

click on the according program if you start it from Windows through the explorer;

Or put in the Floppy-Disk, switch on the PC and wait about 2 Minutes; when a:\ appears input “irrigate” for to load the controller with the Irrigation-Program

input “solar3” for to load the controller with the Original Solar-Cooker-Program

input “temptime” for to load the controller with the Temperature/Time-Program

When there is a message: “cannot find...” then press return until many points are coming at the screen. Then the loading of the software was successful!

Afterwards simply take off the plug of the device and the device runs with the new Software and Soul.

## For professionals: Supervising of the values with the PC

The moisture-values are sended automatically every hour to the PC.

If you want to Supervise them    and evaluate them with Excel do the following Steps:

- connect the cable with the PC but with the side: “PC” at the Solar-Controller (the other way round as loading a Program!) and the other side on the PC. Switch on the device, the Ground-Menu has to come otherwise turn the cable round.

- Go to in Windows 95, 98 to Start / Programs / Accessories / Hyperterminal; Start Hyperterminal, give a name for example “test”. Then select connection over “direct COM1” or “direct COM2” depending on which plug    you put the cable in the PC. If you don’t know start with COM1 and press OK;

-Then input in the next window: 9600, 8, none, 1, Hardware and OK;

- Now all minute the values are coming to the Display

- If you want to have the values in Excel: Go in Hyperterminal to Pull-Down Menu Transmission, select “Record Text” and give an name and a directory you know. Now let this run as long as you want to have the values recorded. Then go to Transmission and select “end Recording” and finish Hyperterminal.

- Now you go to Excel, select File open, select “all files” and then select the file you stored and select “next” in the following window; Then select separated by “space”; and then press “next” and end.

- Now you have a table of the values and you can easy make a diagram from it

- Mark the table, select “add” “Diagram”, select one you like and you have a wonderful Diagram, which you can print out as well.

For Contact:

Stefan Schranner

Franz-Ludwig-Str.2

97072 Würzburg

Tel.: 0049 931 6101 416

after 17 o’clock: 72353

FAX: 0049 931 6101 468

Email: StefanSchranner(at)yahoo.de