Short Manual for LogView

Short Overview about the Feature of Logview	1

Installation	1

Sending of logged Data to LogView	3

# Short Overview about the Features of Logview

View of Process-Data in Graphic

View of Process-Data in Table-Format

Evalate the Data in Real-Time

Comparison of Curves

The Design can be changed

Intuitive workaround

Integrated RTF Editor

Graphic-Editor

Export-Funktions:

Table-Format: CSV, ASCII, HTML, RTF und Excel

Graphic-Format: BMP, JPG, GIF und PNG

# Installation

For to let the Metamorphose send Data to Logview start the Software MP_Remote and select Menu „Data, Send Data for Logging“.

Then finish the Software MP_Remote and Start LogView. Now all relevant Data are send cyclic.

Install Logview with LogViewInstaller from the CD (Folder \LogView) or the latest Version at the Internet: www.logview.info/cms/d_logview-21.phtml

After the Installation of the Software copy from the CD \LogView\OpenFormat\ all files to

C:\Documents and Settings\your User-Name\Application Data\LogView\Geraete\OpenFormat\

Start Labview

Select Device at Menu „Device, choose Device and Port“ the according Device:

Solar Charge Controller Cycle Data: Metamorphose_Solar_e

Solar Charge Controller Flash Data: Metamor_Solar_Flash_e
Irrigation-System Cycle Data: Metamor_Irrigate

Irrigation-System Flash Data: Metamor_Irrigate_flash

Select according serial COM-Port as well

At Menu File, Settings, Program select the following:

In the Default path for Log-Files the Log-Files are stored. You can modify it. These Files can be checked later on just by double-click

Registrier Log suffix should be checked that you can start Logview by direct clicking the *.lov file.

Then you can start the recording. In the Life-Window Data should come up.

If you want to have the actual Time being displayed in the Curves

Select Menu Grafic, Zeitformat der X Achse, then ontop instead of R_ make D_ and save.

Sending of logged Data to LogView

Logged Data stored in the Flash-Memory from Analog-Inputs can be uploaded to Logview. Switch in Menu 8, Parameter 75 to „1“ to upload. After Transfer of all data this value is again „0“. Of course LogView has to be started and as Device Metamor_Solar_Flash has to be selected. 
Hint: The prepared Logview-file Metamor_Solar_Flash works only with two values, Battery-voltage and Solar Current and Parameter 44 has to be set to “2”. Otherwise the File Metamor_Solar_Flash_e  at Logview has to be modified. This can be done by the OpenFormatEditor at

"C:\Program Files\LogView 2\OpenFormat\OpenFormatEditor.exe" erfolgen (also in the Programm-Group).

More Infos in the online-Manual of Logview and the Internet under www.logview.info